PM2.5 in New Zealand

Modelling the current (2018) levels of fine particulate air pollution

Prepared for the Ministry for the Environment

December 2019

Prepared by:

Ian Longley

Guy Coulson

For any information regarding this report please contact:

Ian Longley

Programme Leader - Air Quality

+64-9-375 2096

ian.longley@niwa.co.nz

National Institute of Water & Atmospheric Research Ltd
Private Bag 99940
Viaduct Harbour
Auckland 1010

Phone +64 9 375 2050

NIWA CLIENT REPORT No:	2019319AK

Report date:			December 2019

NIWA Project:			MFE20101

| Quality Assurance Statement |  |  |
| --- | --- | --- |
|  | Reviewed by: | Elizabeth Somervell |
|  | Formatting checked by: | Emma Hope-Ede |
|  | Approved for release by: | Jonathan Moores Regional Manager - Auckland |

Version history

First draft released July 2016 as NIWA CLIENT REPORT No: AKL2016-015

Circulated to central and local government agencies

Feedback and new data incorporated October 2019

This updated version released December 2019

Contents

Executive summary	5

1	Introduction	6

1.1	Background	6

1.2	Purpose and scope of this report	6

1.3	Limitations of this work	6

2	Methods	7

2.1	Overview	7

2.2	Origin, coverage and use of monitoring data	8

2.3	Exposure Model – Conceptual approach	8

2.4	Modelling coarse and natural particles	10

2.5	PM2.5/PM10 ratios	16

2.6	Exposure model validation	16

2.7	Health effects modelling	17

2.8	Likely number of exceedances of a daily PM2.5 standard	18

3	Results	21

3.1	PM2.5 exposure	21

3.2	Exceedances of a daily PM2.5 standard	23

3.3	Health outcomes attributable to PM2.5	25

3.4	Health outcomes attributable to PM10	26

4	Acknowledgements	26

References	27

Appendix A	Source Apportionment data used	28

Appendix B	PM10 exceedances 2015 - 2018	32

Tables

Table 2-1:	Estimated contribution to natural coarse particles in Gore (g m-3).	15

Table 2-2:	Estimated contribution to natural fine particles in Gore (g m-3).	15

Table 3-1:	Predicted annual mean PM2.5 (2018) for all gazetted airsheds, plus natural and anthropogenic contributions.	21

Table 3-2:	Predicted number of likely exceedances of different daily PM2.5 standards.	23

Table 3-3:	Estimated health outcomes for PM2.5 in 2018.	25

Table 3-4:	Results of sensitivity testing of mortality concentration-response functions.	26

Table 3-5:	Estimated health outcomes for PM10 in 2018.	26

Figures

Figure 2-1:	Flow diagram of the risk assessment process.	7

Figure 2-2:	Annual mean marine aerosol contribution to PMcoarse (GNS), and airborne corrosivity (HERA, 2011).	12

Figure 2-3:	Empirical correction function for marine aerosol for east coast sites.	12

Figure 2-4:	Final model of coarse marine aerosol at CAU level.	13

Figure 2-5:	Comparison of modelled and observed coarse PM at a selection of monitoring sites.	17

Figure 2-6:	Relationship between annual mean PM2.5 and likely number of exceedances of a daily standard of 25.	19

Figure 3-1:	Modelled number of exceedances of a PM2.5 standard of 25ugm-3 per year compared to the number of exceedances of the current PM10 standard in 2018.	25

Executive summary

As part of an update of the NESAQ (National Environmental Standards for Air Quality), the Ministry for the Environment (MfE) wished to investigate the introduction of new standards to control concentrations of PM2.5 in ambient air. A baseline of current concentrations and associated health impacts was required as an input to a cost-benefit analysis of various policy options, which will include future projections of PM2.5 levels.

No pre-existing national model of PM2.5 exposure was available for this work. Thus, one had to be created from scratch. Consequently, the model currently stands unvalidated and we anticipate that further improvements may be made. However, we believe that this work best represents the data available now and is fit for purpose.

Once PM2.5 exposures were estimated, health effects were calculated using the same well-established method used in the 2012 Updated HAPINZ study and in subsequent MfE environmental reporting. A required modification was the adoption of an internationally-approved dose-response function for PM2.5 in the absence of one based on New Zealand data.

For 2018 we estimated the following:

The national population-weighted average concentration of PM2.5 is 5.6 g m-3.

The national population-weighted average concentration of PM2.5 from anthropogenic sources is 3.3 g m-3.

11 airsheds have an annual mean PM2.5 concentration of 10 g m-3 or higher covering an approximate population of 228,000.

19 airsheds have an annual mean PM2.5 concentration of 8 g m-3 or higher. These airsheds have an estimated population of around 666,000.

19 airsheds are estimated to exceed a daily mean PM2.5 standard of 25 g m-3.

Anthropogenic PM2.5 exposure is associated with

646 premature deaths in adults per year

215 extra cardiac hospital admissions

422 extra respiratory hospital admissions

1.6 million restricted activity days.

# Introduction

## Background

As part of an update of the NESAQ (National Environmental Standards for Air Quality), the Ministry for the Environment (MfE) wished to investigate the introduction of new standards to control concentrations of PM2.5 in ambient air. A baseline of current concentrations and associated health impacts was required as an input to a cost-benefit analysis of various policy options, which will include future projections of PM2.5 levels.

## Purpose and scope of this report

This report covers the methods used and results for:

Production of estimates of the current concentrations of PM2.5 in unmonitored gazetted airsheds across the country.

Estimation of the contributions of natural and anthropogenic sources to current PM2.5 concentrations for each gazetted airshed.

Estimation of the likely number of exceedances of a daily PM2.5 standard of 25 g m-3, and alternative values.

The use of PM2.5 concentration estimates (monitored where available, otherwise modelled: see section 2.2) to produce estimates of the current health impacts of anthropogenic PM2.5.

An assessment of the current health impacts of anthropogenic PM10.

All assessments are based on the year 2018, or the nearest alternative year for which credible data or estimates can be generated.

This report is accompanied by three spreadsheets providing full results.

This report does not cover the cost-benefit analysis which was conducted by other parties.

## Limitations of this work

No pre-existing national model of PM2.5 was available for this work. Thus, one had to be created from scratch. The model stands unvalidated at present and we anticipate that further improvements could be made. The timeframe also meant that a comprehensive health effects literature search could not be conducted.

# Methods

## Overview

This assessment, like other similar assessments before it, consists of two parts:

Exposure modelling

Health effects modelling

- The relationship between them is illustrated in Figure 2-1. PM10 is only observed at a limited number of locations and PM2.5 in fewer locations. The purpose of the Exposure Model is to allocate estimates of PM to all locations across the country. The Health Effects Model then estimates the fraction of relevant observed health outcomes that can plausibly be attributed to PM exposure.

Figure 2-1:	Flow diagram of the risk assessment process.

- The approach used for the Health Effects Model, popularised by Künzli et al (2000), is highly consistent with international practice, and is identical to that used in the 2012 Updated HAPINZ assessment (Kuschel et al., 2012).

- Exposure models, however, vary in approach across the world in response to the nature and coverage of data available. The 2007 HAPINZ assessment (Fisher et al., 2007) and the Updated 2012 HAPINZ assessment (Kuschel et al., 2012) used different exposure modelling approaches to reflect the different amounts of PM10 data available at the time.  For this work, a growing amount of PM10 data was available, but PM2.5 data was sparse. For this reason, we have adopted some parts of the 2012 Updated HAPINZ Exposure Model but also introduced a new modelling approach to generate plausible estimates of PM2.5.

## Origin, coverage and use of monitoring data

The observed PM data used in this assessment was all collected by regional councils and unitary authorities (with one exception – data from Huntly was collected for Genesis Energy). Other data collected by, or on behalf of, NIWA, NZTA or other industrial concerns have not been included. Data were provided to us by MfE. Additional PM10 data were provided by Auckland Council, Bay of Plenty Regional Council, Environment Southland and Environment Canterbury.

For inclusion in this assessment we required that any measure of annual mean PM10 had to meet a 75 % data coverage criterion, as required by the NESAQ and consistent with the 2014 Air Domain Report and Our Air 2018 (MfE & StatsNZ, 2014, 2018). 2018 data was used as a priority. Where it was not available data from the most recent year available was used, using either data directly provided by councils, the 2014 Air Domain Report, Our Air 2018 or the 2012 Updated HAPINZ exposure model.

For PM2.5 data meeting this criterion for 2018 were available from 18 sites, each representing a separate airshed: four in Auckland, four in Greater Wellington, one in Marlborough, and nine in Canterbury. Due to the scarcity and lack of geographical coverage, we relaxed the 75 % criterion. Where an annual mean was not available for a full calendar year but > 6 months of data was available including at least 2 months of winter data, an annual mean was estimated (Tokoroa, Nelson A and Nelson B). Further historical data or data fragments were available from Auckland (four sites), Hamilton, New Plymouth, Hastings, Richmond and Dunedin.

PM10 data was available from 52 locations in 2018, covering 37 gazetted airsheds and three locations outside gazetted airsheds. Historic data was available from 98 locations, (covering 53 gazetted airsheds and 21 locations outside gazetted airsheds).

Modelling is based on 2013 CAUs, which are then aggregated up to airshed level for policy development. Each of the 16 regions in New Zealand is considered as an airshed. Regional councils and unitary authorities may define specific areas for management within their region by notice in the NZ Gazette to be a separate airshed. There are currently 73 gazetted airsheds in New Zealand (https://data.mfe.govt.nz/layer/98617-nz-airsheds-gazetted/). The airsheds from which data are available to be used in this assessment are therefore only a subset of all airsheds in New Zealand.

## Exposure Model – Conceptual approach

The Exposure Model has two components:

Generation of PM2.5 estimates for locations where PM monitoring is being conducted, or has in the recent past

Allocation of PM2.5 estimates to locations where no monitoring has ever been conducted (both within gazetted airsheds, and outside gazetted airsheds).

### Generating PM2.5 estimates

PM2.5 is a subset of PM10 such that PM10 = PM2.5 + PMcoarse. International research, and local studies by the Institute of Geological and Nuclear Science (GNS Science) (see for example Appendix A), show  that coarse particulate matter (PMcoarse, also known as PM2.5-10) is, in most locations, dominated by particles with natural sources (sea salt, soil and mineral dust) whose spatial variation is largely determined by physical geography. The most prevalent anthropogenic component of PMcoarse is generally considered to be road dust.

To estimate PM2.5 we considered two approaches:

a ‘direct’ approach in which PM2.5 is estimated as a function of local emissions and meteorology,

an ‘indirect’ approach in which PMcoarse is estimated as a function of geographical proxies and the result subtracted from observed PM10.

With a very limited timeframe, we chose the indirect approach after a consideration of the data availability and likely errors involved in each approach. In future we recommend that both approaches are developed further, and their differences resolved so that their relative merits and limitations are better understood.

Thus, we took the following conceptual approach.

Where PM2.5 is measured, those data will be used

PMcoarse and the natural contribution to PM2.5 will be modelled for every CAU in the country. This modelling will be based, in part, on compositional data generated by GNS (Appendix A) and is described in more detail in section 2.4.

Where PM10 is measured but PM2.5 is not, PM2.5 will be estimated by subtracting the modelled PMcoarse estimate from observed PM10

- This process results in estimates of PMcoarse and the natural contribution to PM2.5 for every CAU in the country and estimates of total PM2.5 for every CAU where PM2.5 or PM10 monitoring data are available. This approach allows us to estimate the anthropogenic and natural components of PM from a bottom-up summing of separately modelled components, rather than a top-down splitting up of a total PM estimate. We believe our approach generates more realistic estimates of this split in locations where it has not been observed (>99% of the country).

### Allocating PM2.5 estimates to unmonitored CAUs

For this task we largely followed the approach of the 2012 Updated HAPINZ Exposure Model, with some modifications.

The natural component of PM2.5 is already estimated for every CAU. Therefore, it is only the anthropogenic component which is allocated.

In airsheds where a PM2.5 estimate has been generated, the estimate for the anthropogenic component is applied to all CAUs in the airshed. This is then added to the estimated natural component for each CAU.

In the multi-town airsheds of Otago, a single PM2.5 value has been allocated to all CAUs in all towns constituting the airshed (a decision made in agreement with MfE). This introduces a known error but can easily be changed.

There appears to be no official definition of which CAUs lie in which airsheds as most airshed boundaries cut across CAUs. We have taken a relatively conservative approach and allocated airshed PM2.5 estimates to CAUs at the edge of the airshed which are substantially rural. This affects a population of approximately 200,000.

To exploit the higher density of monitoring data and attempt to capture spatial variation in the Auckland Region, we have adapted the method of the 2012 Updated HAPINZ model, i.e. a simple linear regression between emission density per CAU and concentrations observed within that CAU (adapted to use estimated anthropogenic contribution to PM2.5).

Towns where no qualifying PM monitoring has ever been conducted are allocated an anthropogenic PM2.5 estimate in the same manner as the 2012 Updated HAPINZ model, i.e. by allocating data from other towns in the same region, or by allocating estimated rural or small-town values.

## Modelling coarse and natural particles

### The GNS Science dataset

For over a decade GNS Science has been collecting air particulate filter samples from monitoring stations around the country on behalf of some regional councils, and as part of a research programme. Ion beam analysis is used to determine elemental composition. Principal Component Analysis (PCA) was used to provide an initial indication of the number of sources that may be contributing to the sample. Positive Matrix Factorisation (PMF) was used to apportion mass contributions and determine relative uncertainties and closeness of fit of the model to the data. This approach is used successfully for source apportionment studies by researchers in New Zealand and overseas.

For the purposes of this work, monthly summary data were provided to NIWA by GNS Science. This dataset covers 23 monitoring sites, including two in Alexandra, seven in Auckland and five in Greater Wellington. It consists of PM10 data at 20 sites and PM2.5 at 16 sites. NIWA has subsequently used these data to calculate annual mean contributions for several key recurring sources (Appendix A). In doing so we acknowledge that this method introduces some error. The attribution of certain elemental combinations to a given source is location-specific related to local source activity, thus generalisations must be made carefully. Some sources can be distinguished in some places and not in others (the power to disaggregate sources generally improves with the number of filters analysed).

For this work we mainly make use of the contribution estimates for coarse particles of natural source. Data for PMcoarse is available from 14 sites – seven of which are direct measurements, and seven of which are calculated from PM10 – PM2.5. The most important natural coarse particle sources are marine aerosol, and “soil” particles. These two classes of particles are generally quite compositionally distinct which allows them to be separately resolved at all but one of the 14 sites. Source definition is aided, in some cases, by meteorological analysis (especially analysis by wind direction) where a site is influenced by different sources in different wind directions. However, the “soil” class is likely to include a range of different types of particles from different sources, including wind-blown natural dusts (particularly gravel and top-soil), soils disturbed by agricultural practices and quarrying. Industrial processes may also emit ‘soil’-like particles as may construction, although these may be mixed with combustion products. At four sites a “road dust” contribution – which may incorporate dusts from unsealed roads and yards and vehicle wear products) is separately resolved, largely because of the association with combustion-related chemicals, whereas at others it is assumed to be included in the “soil” source, but the contribution cannot be quantified. Furthermore, a “fertiliser” source is also resolved at Dunedin and Patumahoe which is tentatively related to specific local sources such as handling and use of fertiliser. In practice this means that the particles included in an estimate of “soil” varies from site to site and it is very difficult to specify how much of that soil contribution is generally representative of the region, and how much is highly local. Nevertheless, we believe that this source apportionment approach is of enough power to be valuable.

### Marine aerosol

Filter analysis work by GNS Science has indicated that marine aerosol presents the largest contribution of any identified source to PMcoarse at 12 out of 13 locations assessed contributing 3 – 7 g m-3 as an annual average (Appendix A), or 23 - 75 % of PMcoarse and 14 – 40 % of PM10. GNS data also shows marine aerosol contributes 0 – 3 g m-3 to PM2.5 where it has been identified. The impact of the transport of airborne sea salt inland in New Zealand has previously been characterised in terms of the potential for corrosion, which has been mapped at high resolution across the country (HERA, 2011). We extracted estimates of corrosivity for the air monitoring locations from where the GNS data originate. We found a good correlation between annual mean marine aerosol concentrations and airborne corrosivity for all west-facing coasts and inland sites. A linear parameterisation was generated (Figure 2-2) and subsequently applied to all CAUs in the exposure model. A cap of 11 g m-3 was introduced to prevent predicted PMcoarse exceeding observed PM10 in the most-affected monitoring site of Westport.

For sites within ~10 km of east-facing coasts the marine aerosol was lower than that predicted using the west-facing coast model. We hypothesise that this is related to either the lower prevalence of easterly winds, or the typically lower wind speeds on the east coast. We recommend that this is explored further in future. However, for this work we generated an empirical adjustment to the west-facing coast model as a function of distance to an east-facing coast (Figure 2-3). This was based on marine contribution to PM10 rather than PMcoarse, due to more data being available and a better empirical fit. We therefore reduced the adjustment by a further 25 % as 75 % of PM10 marine aerosol is typically in the coarse fraction.

Figure 2-2:	Annual mean marine aerosol contribution to PMcoarse (GNS), and airborne corrosivity (HERA, 2011).

Figure 2-3:	Empirical correction function for marine aerosol for east coast sites.

The marine aerosol contribution to PM2.5, which is required so that PM2.5 can be apportioned between natural and anthropogenic fractions, was also estimated through correlation with corrosivity followed by a correction for east coast sites.

Figure 2-4:	Final model of coarse marine aerosol at CAU level.

### Non-marine contributions to coarse particles

Data from GNS Science indicate that, after marine aerosol, the next most prevalent source contribution to PMcoarse that has been identified is ‘soil’ (see Appendix A). There is some ambiguity as to what this source profile precisely represents but is likely to include agricultural soils resuspended into the air by the wind, but also by mechanical action (machinery and road transport). It may also include soils or mineral dusts resuspended by quarrying, construction, demolition, from dust-emitting industries, etc. An additional anticipated, but poorly quantified source is dust resuspended by the wind from braided river valleys. In some analyses GNS Science have resolved an independent source profile relating to road dust or a local industry.

In general, coarse particles have a reduced atmospheric residence time relative to finer particles. This gives their airborne concentrations strong spatial gradients, and which are responsive to highly local sources, i.e. concentrations vary significantly over hundreds, even tens of metres. Consequently, there is a large degree of variability in source contributions, and how likely they are to be resolved by source apportionment techniques, relative to the small number of observations available. This makes generalisation across the country very challenging.

In view of this we took a relatively subjective approach. We estimated the non-marine contribution to PMcoarse at a range of locations (by subtracting estimated or observed coarse marine aerosol from observed PMcoarse) and found moderately higher concentrations in Canterbury and Otago.

We hypothesised that part of the soil contribution would be related to surface dryness. We trialled correlating the non-marine contribution to PMcoarse with the number of wet days per year at the monitoring location. The correlation was insufficiently strong to justify modelling any relationship across the country. Alternatively, we applied the following simple estimates for ‘soil’ contribution:

3 g m-3 in Canterbury and Otago Regions

1 g m-3 in all other regions

To account for the observably higher non-marine contribution to PMcoarse at urban sites, which we hypothesise is primarily due to road dust, we assumed a relationship with motor vehicle emissions. A linear relationship was empirically derived which contributed 0 – 1.6 g m-3 of PMcoarse across the country.

Other contributions to PMcoarse were not modelled. Contributions identified in GNS Science source apportionment include biomass burning (at Masterton and Raumati South only), sulphate (at Lower Hutt only) and fertiliser soil (at Dunedin and Patumahoe only). It is not possible currently to further specify the origin of the ‘biomass burning’ component. It may, or may not include woodsmoke from domestic heating, open or agricultural burning, or industrial sources. We hold that there is insufficient data currently available to include these contributions, which may be highly localised, in a national model.

Data on the non-marine contribution to PM2.5 is even scarcer. Based on a review of GNS Science data we opted to apply an estimate of 1.4 g m-3 (made up of 1 g m-3 sulphate and 0.4 g m-3 soil) across the whole country.

### Estimation of anthropogenic and natural contributions to PM

The natural and anthropogenic PM concentrations were calculated as below (bold = modelled unless observed, italic = observed or allocated):

Natural PM2.5 = marine PM2.5 + soil & sulphate PM2.5

Natural PMcoarse = marine PMcoarse + soil PMcoarse

Anthropogenic PM10 = PM10 – (Natural PM2.5 + Natural PMcoarse)

PMcoarse = Natural PMcoarse + Urban PMcoarse

PM2.5 = PM10 - PMcoarse

Anthropogenic PM2.5 = PM2.5 – Natural PM2.5

### Worked example 1: Gore (example of small airshed with PM10 monitoring)

The Gore airshed consists of seven census area units. Environment Southland runs a PM10 monitoring site in the town. In 2018 the annual mean PM10 concentration was 18.5 g m-3. The contribution of natural coarse particles to this concentration is estimated for the CAU in which the monitoring site is situated, and all other CAUs in the airshed.

Table 2-1:	Estimated contribution to natural coarse particles in Gore (g m-3).

|  | Monitoring site | All airshed |
| --- | --- | --- |
| Coarse marine aerosol | 2.7 | 2.6 – 3.0 |
| Dust | 1.0 | 1.0 |
| Urban dust (Urban PMcoarse) | 0.4 | 0.1 – 0.4 |
| Total PMcoarse | 4.1 | 3.8 – 4.1 |

We therefore estimate PM2.5 as PM10 – PMcoarse = 18.5 – 4.1 = 14.4 g m-3 at the Gore monitoring site.

The natural contribution to PM2.5 is estimated as in Table 2-2.

Table 2-2:	Estimated contribution to natural fine particles in Gore (g m-3).

|  | Monitoring site | All airshed |
| --- | --- | --- |
| Fine marine aerosol | 0.5 | 0.5 – 0.6 |
| Soil & sulphate | 1.4 | 1.4 |
| Total natural PM2.5 | 1.9 | 1.9 – 2.0 |

Thus, the anthropogenic contribution to PM2.5 is calculated as PM2.5 – natural PM2.5 = 14.4 – 1.9 = 12.5 g m-3.

This estimate is then assumed to be constant for every CAU in the Gore airshed and summed to the individually modelled natural PM2.5 (1.9 – 2.0 g m-3) for each CAU to provide an estimate of total PM2.5 for every CAU.

Finally, the PM2.5/PM10 ratio for Gore is taken from the values for the CAU containing the monitoring site, i.e. 14.4 / 18.5 = 0.78.

### Worked example 2: Thames (example of small airshed with no PM10 monitoring)

The method is the same as example 1, except that no PM10 observation exists. In this case a PM10 value is allocated using a method adapted from the 2012 Updated HAPINZ Exposure Model.

In the example of Thames, this is classed as a ‘Type 3’ urban rural classification (according to Stats NZ). The HAPINZ method allocates an anthropogenic PM2.5 estimate to all Type 3 CAUs in the Waikato Region which is the lowest anthropogenic PM2.5 concentration modelled in Type 1, 2 or 3 CAUs across the region. This happens to be 2.9 g m-3 at Pukekapia Rd, Huntly, which is then summed with the modelled estimate for natural PM2.5 at Thames (2.7 g m-3) to give 5.6 g m-3.

## PM2.5/PM10 ratios

MfE requested that we provide estimated values of the PM2.5/PM10 ratio for each airshed. This was done simply by dividing the estimated annual mean PM10 concentration by the estimated annual mean PM2.5 concentration, as both concentrations were outputs of the exposure modelling.

## Exposure model validation

There are limited options available for validation of the exposure model. All available observational PM2.5 data has been used for model training.

However, the spatial model for PMcoarse can be partially validated. We have collated observed annual mean PMcoarse data (PM10 – PM2.5) from regional council monitoring for 2018. Sites where there is a known, local and unmodelled contribution to PMcoarse (mainly industrial) should not be included in the validation. This includes Dunedin (where the monitoring site appeared to be locally impacted by ≈5 g m-3 of fertiliser-sourced PMcoarse during sampling, according to GNS Science data), Woolston (local industry, impacted by an unusually high soil contribution according to GNS Science data), Nelson B (local industry) and Washdyke (local industry). The degree to which other locations may be similarly impacted by such localised sources is unknown and remains a limitation of our modelling approach.

The resulting data (16 sites, Figure 2-5) shows that our model predicts PMcoarse to within 1.2 g m-3 at all sites except Auckland - Queen Street and Wellington Central - Willis Street, where the model over-estimates by 1.6 g m-3. The model under-estimates by 2 - 4 g m-3 at Woolston, Washdyke and Nelson B (not shown). These disagreements may also be due to shipping emissions (e.g. coarse sulphate), unsealed yards, earth-moving activities, etc., uncertainty in defining the distance to coast, the significance of elevation or differential deposition to urban surfaces within complex natural or built topography.

Figure 2-5:	Comparison of modelled and observed coarse PM at a selection of monitoring sites.

## Health effects modelling

The method used is identical to the 2014 Air Domain Report and with only minor changes from the 2012 Updated HAPINZ assessment. We used the same health baseline data, used 2013 population data, and assessed the impact of the anthropogenic fraction only.

The major change that was required was to calculate health outcomes associated with PM2.5 in addition to PM10.

We conducted a limited review of available concentration-response functions. The 2012 HAPINZ assessment prioritised used of a local function for mortality (7% per 10 g m-3 of PM10, Hales et al., 2010) over more commonly used international functions (such as 4.3 % per 10 g m-3 of PM10, e.g. Künzli et al., 2000). For consistency with previous assessments in New Zealand, and in the absence of any new NZ-based function, we continued to use this function for this assessment.

There is, however, no equivalent New Zealand function for PM2.5. Internationally, a function of 6.2% per 10 g m-3 of PM2.5 is recommended by both the WHO (2013) and COMEAP (2009). We chose to adopt this function for our work as we consider it to be the most appropriate and robust approach.

Alternative approaches considered but rejected were:

Estimate an equivalent of the Hales et al. (2010) function for PM2.5, based on observed PM2.5/PM10 ratios. However, our exposure modelling shows that this ratio is highly spatially variable in New Zealand (varying from approx. 20% - 90%), plus the 7% figure was based on a New Zealand’s first PM exposure model which more recent monitoring and our modelling suggests contains significant errors.

Use of a function of 17% for combustion-derived PM2.5 particles based on a study of Los Angeles by Jerrett et al. (2005) and adopted in a Swedish national health risk assessment (Gustafsson et al., 2014). While we feel this approach has scientific merit, and could be applied in New Zealand, we also note that there is limited precedent (we found no other jurisdiction using this approach so far) and we recognise that the composition of combustion particles in most of New Zealand (dominated by woodsmoke) is substantially different to that in Los Angeles (traffic/industrial).

Independent assessment of the health impacts associated with the coarse fraction PM2.5 – 10. There is very limited and inconsistent international evidence of a robust association, especially representing the composition of coarse particles in New Zealand (dominated by marine aerosol). The WHO does not recommend any concentration-response function for coarse particles. In the Swedish assessment (Gustafsson et al., 2014) a function of 17 % was used for the road dust component of PM10 based on a health effects study in Stockholm where the use of studded tyres in winter leads to far higher concentrations than observed in New Zealand. Other coarse particles were not assessed. A review of the health effects of coarse particulate matter conducted by Health Canada recently concluded

“In regard to the chronic effects of coarse PM, the health database is inadequate to infer a causal relationship with mortality, respiratory and cardiovascular health outcomes, as well as with the incidence of developmental outcomes.” (Health Canada, 2016)

- Other functions for PM2.5 adopted followed the recommendations of the WHO (2013) were:

0.0091 cardiac hospital admission (all ages) per person per year per 10 g m-3 daily PM2.5

0.019 respiratory hospital admission (all ages) per person per year per 10 g m-3 daily PM2.5

0.9 restricted activity days (all ages) per person per year per 10 g m-3 daily PM2.5

## Likely number of exceedances of a daily PM2.5 standard

- An additional requirement for cost-benefit analysis was a prediction of the likelihood that there will be exceedances of a potential future daily PM2.5 standard.

- We considered that there was too little observational data to establish this through direct analysis alone. Most of the PM2.5 data available is from Auckland where exceedances of likely daily standards (25 g m-3 and above) do not, or rarely occur.

- To tackle this issue, we instead took two approaches. Firstly, we reviewed the factors influencing the number of daily exceedances of the PM10 standard. Secondly, we created thousands of simulated daily PM2.5 time series calibrated to broadly match the statistical characteristics of real PM2.5 time series.

- Through this work we were able to establish that the likely number of exceedances of any daily standard are related to

The annual mean

The duration of winter

The mean summertime concentration

- We found that the limited amount of PM2.5 data available at this time meant that we could quantify only the first relationship. Specifically, we fitted a logarithmic relationship:

- Number of exceedances = A ln(mean PM2.5) + B

- Where A and B are functions of the daily standard limit value and the duration of winter.

- This is illustrated in Figure 2-6. Deviations of observed data from the band created by the simulated results (grey) can be attributed by lower-than-simulated (points to the left) of higher-than-simulated (points to the right) summer average concentrations.

Figure 2-6:	Relationship between annual mean PM2.5 and likely number of exceedances of a daily standard of 25.

Grey = simulation results, green = Timaru, light blue = St Albans, yellow = Woolston, dark blue = Takapuna.

- In this work we have not implemented any influence of duration of winter. Exploratory work has indicated that the (currently not implemented) influence of summertime concentration could be significant. Summer concentrations are dominated by motor vehicle, marine and secondary sulphate sources. This omission remains an outstanding limitation of our method and estimates to date. Nevertheless, we believe our model represents the best understanding available currently.

# Results

## PM2.5 exposure

We estimated that

The national population-weighted average concentration of PM2.5 in 2018 is 5.6 g m-3.

The national population-weighted average concentration of PM2.5 from anthropogenic sources in 2018 is 3.3 g m-3.

11 airsheds have an annual mean PM2.5 concentration of 10 g m-3 or higher covering an approximate population of 228,000.

19 airsheds have an annual mean PM2.5 concentration of 8 g m-3 or higher covering an approximate population of 666,000.

Estimates for gazetted airsheds are provided in Table 3-1.

Table 3-1:	Predicted annual mean PM2.5 (2018) for all gazetted airsheds, plus natural and anthropogenic contributions.

| Represented airshed | Representing monitoring site | PM2.5 / g m-3 |  |  | PM2.5/PM10 ratio |
| --- | --- | --- | --- | --- | --- |
|  |  | Total | Natural | Anthropogenic |  |
| Invercargill | Invercargill Pomona | 17.2 | 1.6 | 15.6 | 0.82 |
| Richmond | Richmond | 15.3 | 1.5 | 13.8 | 0.82 |
| Gore | Gore | 14.4 | 1.9 | 12.5 | 0.78 |
| Taihape | Taihape | 14.4 | 1.8 | 12.5 | 0.81 |
| Blenheim | Blenheim Redwoodtown | 13.2 | 2.2 | 11.0 | 0.74 |
| Otago 2 | Mosgiel | 13.1 | 1.9 | 11.2 | 0.68 |
| Awatoto | Awatoto | 11.5 | 3.3 | 8.2 | 0.58 |
| Nelson A | Nelson Airshed A | 11.3 | 1.5 | 9.8 | 0.67 |
| Timaru | Timaru | 10.8 | 2.3 | 8.5 | 0.55 |
| Tokoroa | Tokoroa | 10.2 | 2.0 | 8.2 | 0.69 |
| Masterton | Masterton East | 10.0 | 2.0 | 8.0 | 0.70 |
| Otago 1 | Alexandra | 9.9 | 1.7 | 8.2 | 0.70 |
| Kaiapoi | Kaiapoi | 9.9 | 1.9 | 8.0 | 0.60 |
| Geraldine | Geraldine | 9.6 | 2.1 | 7.5 | 0.60 |
| Rotorua | Rotorua Edmond Road | 9.1 | 2.0 | 7.1 | 0.67 |
| Ashburton | Ashburton | 8.9 | 2.1 | 6.8 | 0.56 |
| Te Kuiti | Te Kuiti | 8.5 | 2.1 | 6.4 | 0.64 |
| Warkworth | Warkworth | 8.2 | 1.5 | 6.7 | 0.47 |
| Christchurch | Christchurch St Albans | 8.1 | 2.3 | 5.8 | 0.47 |
| Napier | Napier | 7.9 | 2.1 | 5.8 | 0.61 |
| Waimate | Waimate | 7.8 | 2.0 | 5.8 | 0.56 |
| Nelson B | Nelson Airshed B | 7.6 | 2.5 | 5.1 | 0.42 |
| Ngaruawahia | Ngaruawahia | 7.5 | 2.3 | 5.2 | 0.57 |
| Taupo | Taupo | 7.5 | 1.9 | 5.6 | 0.67 |
| Otago 3 | Dunedin | 7.4 | 2.2 | 5.2 | 0.49 |
| Rangiora | Rangiora | 7.4 | 2.2 | 5.2 | 0.48 |
| Matamata | Matamata | 7.3 | 2.2 | 5.1 | 0.58 |
| Te Awamutu and Kihikihi | Te Awamutu | 7.0 | 2.1 | 4.9 | 0.59 |
| Turangi | Turangi | 6.8 | 1.8 | 5.0 | 0.68 |
| Cambridge | Cambridge | 6.7 | 2.0 | 4.7 | 0.61 |
| Whangarei | Whangarei | 6.6 | 2.9 | 3.7 | 0.47 |
| Putaruru | Putaruru | 6.6 | 2.0 | 4.5 | 0.59 |
| Hamilton City | Hamilton | 6.4 | 2.2 | 4.2 | 0.55 |
| Huntly | Huntly Croft Tce | 6.4 | 2.2 | 4.2 | 0.56 |
| Taumarunui | Taumarunui | 6.4 | 2.0 | 4.4 | 0.60 |
| Auckland Urban | Takapuna | 6.3 | 2.5 | 3.8 | 0.47 |
| Hastings | Hastings | 6.1 | 2.3 | 3.8 | 0.45 |
| Tuakau | Tuakau | 5.9 | 2.3 | 3.6 | 0.39 |
| Pukekohe | Pukekohe | 5.9 | 2.3 | 3.6 | 0.39 |
| Porirua | Porirua | 5.9 | 2.5 | 3.4 | 0.48 |
| Wainuiomata | Wainuiomata | 5.7 | 2.2 | 3.5 | 0.52 |
| Thames | Thames | 5.6 | 2.7 | 2.9 | 0.47 |
| Waihi | Waihi | 5.6 | 2.5 | 3.1 | 0.48 |
| Upper Hutt | Upper Hutt | 5.5 | 2.1 | 3.4 | 0.53 |
| Wellington City | Wellington Central | 5.5 | 2.9 | 2.6 | 0.44 |
| Dargaville | Dargaville | 5.5 | 2.5 | 2.9 | 0.47 |
| Waiuku | Waiuku | 5.4 | 3.3 | 2.1 | 0.38 |
| Whangamata | Whangamata | 5.3 | 2.4 | 2.9 | 0.48 |
| Kapiti Coast | Raumati South | 5.3 | 3.3 | 2.0 | 0.41 |
| Te Aroha | Te Aroha | 5.3 | 2.3 | 2.9 | 0.49 |
| Lower Hutt | Lower Hutt | 5.2 | 2.4 | 2.9 | 0.46 |
| Paeroa | Paeroa | 5.1 | 2.2 | 2.9 | 0.50 |
| Morrinsville | Morrinsville | 5.1 | 2.2 | 2.9 | 0.50 |
| Otorohanga | Otorohanga | 5.1 | 2.2 | 2.9 | 0.50 |
| Kaitaia | Kaitaia | 5.1 | 2.4 | 2.7 | 0.47 |
| Karori | Karori | 4.8 | 2.2 | 2.6 | 0.48 |
| Whitianga | Whitianga | 4.8 | 1.9 | 2.9 | 0.53 |
| Helensville | Helensville | 4.8 | 2.5 | 2.3 | 0.36 |
| Reefton | Reefton | 4.7 | 3.3 | 1.4 | 0.38 |
| Kerikeri | Kerikeri | 4.5 | 1.8 | 2.7 | 0.51 |
| Beachlands-Maraetai | Beachlands | 4.4 | 2.5 | 1.8 | 0.36 |
| Waiheke Island | Waiheke | 4.3 | 3.3 | 1.0 | 0.35 |
| Riverhead | Riverhead | 4.1 | 3.0 | 1.2 | 0.34 |
| Wellsford | Wellsford | 4.0 | 2.6 | 1.3 | 0.35 |
| Nelson C | Nelson Airshed C | 3.8 | 1.6 | 2.2 | 0.50 |
| Snells Beach | Snells Beach | 3.8 | 2.5 | 1.2 | 0.36 |
| Kumeu | Kumeu | 2.9 | 1.8 | 1.2 | 0.19 |

## Exceedances of a daily PM2.5 standard

Table 3-2 lists the predicted number of exceedances of three possible daily standards. It should be noted that these are ‘likely’ exceedances, i.e. a central estimate based on average meteorological conditions and typical emissions. Actual exceedances may be higher or lower. In particular, the number of exceedances (related to peak daily concentrations) are highly sensitive to complex and locally-specific variations in wind patterns and the thermal structure of the atmosphere and are very difficult to predict. These estimates incorporate a known but currently unquantified error relating to the influence of summertime concentrations and duration of winter.

Table 3-2:	Predicted number of likely exceedances of different daily PM2.5 standards.

| Represented airshed | Representing monitoring site | daily PM2.5 limit value (g m-3) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20 | 25 | 30 |
| Invercargill | Invercargill Pomona | 100 | 89 | 75 |
| Richmond | Richmond | 84 | 74 | 61 |
| Gore | Gore | 76 | 66 | 53 |
| Taihape | Taihape | 76 | 66 | 53 |
| Blenheim | Blenheim Redwoodtown | 65 | 56 | 44 |
| Awatoto | Awatoto | 48 | 40 | 29 |
| Nelson A | Nelson Airshed A | 48 | 40 | 29 |
| Timaru | Timaru | 34 | 27 | 17 |
| Tokoroa | Tokoroa | 34 | 27 | 17 |
| Otago 1 | Alexandra | 33 | 26 | 16 |
| Masterton | Masterton | 31 | 24 | 14 |
| Kaiapoi | Kaiapoi | 29 | 22 | 12 |
| Geraldine | Geraldine | 27 | 20 | 10 |
| Rotorua | Rotorua Edmond Road | 26 | 19 | 10 |
| Ashburton | Ashburton | 18 | 12 | 2 |
| Otago 2 | Mosgiel | 15 | 9 | 0 |
| Te Kuiti | Te Kuiti | 13 | 7 | 0 |
| Napier | Napier | 9 | 3 | 0 |
| Warkworth | Warkworth | 8 | 3 | 0 |
| Auckland Urban | Takapuna | 0 | 0 | 0 |
| Beachlands-Maraetai | Beachlands | 0 | 0 | 0 |
| Cambridge | Cambridge | 0 | 0 | 0 |
| Christchurch | Christchurch St Albans | 5 | 0 | 0 |
| Dargaville | Dargaville | 0 | 0 | 0 |
| Hamilton City | Hamilton | 0 | 0 | 0 |
| Hastings | Hastings | 0 | 0 | 0 |
| Helensville | Helensville | 0 | 0 | 0 |
| Huntly | Huntly Croft Tce | 0 | 0 | 0 |
| Kaitaia | Kaitaia | 0 | 0 | 0 |
| Kapiti Coast | Raumati South | 0 | 0 | 0 |
| Karori | Karori | 0 | 0 | 0 |
| Kerikeri | Kerikeri | 0 | 0 | 0 |
| Kumeu | Kumeu | 0 | 0 | 0 |
| Lower Hutt | Lower Hutt | 0 | 0 | 0 |
| Matamata | Matamata | 0 | 0 | 0 |
| Morrinsville | Morrinsville | 0 | 0 | 0 |
| Nelson B | Nelson Airshed B | 0 | 0 | 0 |
| Nelson C | Nelson Airshed C | 0 | 0 | 0 |
| Ngaruawahia | Ngaruawahia | 0 | 0 | 0 |
| Otorohanga | Otorohanga | 0 | 0 | 0 |
| Otago 3 | Dunedin | 0 | 0 | 0 |
| Paeroa | Paeroa | 0 | 0 | 0 |
| Porirua | Porirua | 0 | 0 | 0 |
| Pukekohe | Pukekohe | 0 | 0 | 0 |
| Putaruru | Putaruru | 0 | 0 | 0 |
| Rangiora | Rangiora | 0 | 0 | 0 |
| Reefton | Reefton | 0 | 0 | 0 |
| Riverhead | Riverhead | 0 | 0 | 0 |
| Snells Beach | Snells Beach | 0 | 0 | 0 |
| Taumarunui | Taumarunui | 0 | 0 | 0 |
| Taupo | Taupo | 0 | 0 | 0 |
| Te Aroha | Te Aroha | 0 | 0 | 0 |
| Te Awamutu and Kihikihi | Te Awamutu | 0 | 0 | 0 |
| Thames | Thames | 0 | 0 | 0 |
| Tuakau | Tuakau | 0 | 0 | 0 |
| Turangi | Turangi | 0 | 0 | 0 |
| Upper Hutt | Upper Hutt | 0 | 0 | 0 |
| Waiheke Island | Waiheke | 0 | 0 | 0 |
| Waihi | Waihi | 0 | 0 | 0 |
| Waimate | Waimate | 2 | 0 | 0 |
| Wainuiomata | Wainuiomata | 0 | 0 | 0 |
| Waiuku | Waiuku | 0 | 0 | 0 |
| Wellington City | Wellington Central | 0 | 0 | 0 |
| Wellsford | Wellsford | 0 | 0 | 0 |
| Whangamata | Whangamata | 0 | 0 | 0 |
| Whangarei | Whangarei | 0 | 0 | 0 |
| Whitianga | Whitianga | 0 | 0 | 0 |

Figure 3-1 shows the expected number of exceedances of a daily PM2.5 standard of 25g m-3 compared with current exceedances of the NESAQ daily PM10 standard of 50 g m-3. A list of exceedances of the NESAQ PM10 standard are given in Appendix B.

Figure 3-1:	Modelled number of exceedances of a PM2.5 standard of 25ugm-3 per year compared to the number of exceedances of the current PM10 standard in 2018.

## Health outcomes attributable to PM2.5

Table 3-3:	Estimated health outcomes for PM2.5 in 2018.

| RESULTS |  | Health Effects All Anthropogenic Sources (cases) |
| --- | --- | --- |
| Mortality |  |  |
| Mortality: 30+ yrs |  | 646 |
|  | Total Mortality |  |
| Morbidity |  |  |
| Cardiac Hospital Admissions: All ages |  | 215.3 |
| Respiratory Hospital Admissions: All ages |  | 422.4 |
| Restricted Activity Days |  | 1,600,647 |

Table 3-4:	Results of sensitivity testing of mortality concentration-response functions.

0.04 and 0.083 values recommended by the WHO (2013). Other values have been chosen to represent a range of possible outcomes.

| CRF | mortality cases per annum |
| --- | --- |
| 0.02 | 221 |
| 0.04 | 439 |
| 0.062 | 646 |
| 0.083 | 841 |
| 0.11 | 1,077 |

## Health outcomes attributable to PM10

Health outcomes attributable to PM10 were also assessed in this work. Great care should be taken in comparing these results to those estimated for PM2.5. They were calculated using different concentration-response functions from different origins (PM10 using the NZ-based work of Hales et al., 2010, PM2.5 using a much larger but non-NZ body of evidence summarised by the WHO (2013) and COMEAP (2009). Both estimates, whilst in our view the best available, contain considerable uncertainty. In view of this uncertainty, it should not be inferred that the difference between the PM10 and PM2.5 health effects estimates equates to the health impacts of coarse particles (PM2.5-10).

Table 3-5:	Estimated health outcomes for PM10 in 2018.

| RESULTS |  | Health Effects All Anthropogenic Sources (cases) |
| --- | --- | --- |
| Mortality |  |  |
| Mortality: 30+ yrs |  | 724 |
|  | Total Mortality |  |
| Morbidity |  |  |
| Cardiac Hospital Admissions: All ages |  | 142.6 |
| Respiratory Hospital Admissions: All ages |  | 225.1 |

# Acknowledgements

We would like to thank Perry Davy of GNS Science and Travis Ancelet, formally of GNS Science for the timely provision of data, peer review and assistance. Furthermore, our thanks to Deborah Read (Ministry of Health) for additional comments on an early draft.

#### References

COMEAP (2009). COMEAP: Long-term exposure to air pollution: effect on mortality. A report by the Committee on the Medical Effects of Air Pollutants. ISBN 978-0-85951-640-2

Fisher, G.; Kjellstrom, T.; Kingham, S.; Hales, S.; Shrestha, R. (2007). Health and air pollution in New Zealand (HAPiNZ). Main Report.

Gustafsson, M, Forsberg, B, Orru, H, Åström, S, Tekie, H, Sjöberg, K (2014). Quantification of population exposure to NO2, PM2.5 and PM10 and estimated health impacts in Sweden 2010. IVL Report B 2197

Hales S, Blakely T, Woodward A. (2010). Air pollution and mortality in New Zealand: cohort study, Journal of Epidemiology and Community Health. doi:10.1136/jech.2010.112490.

Health Canada, 2016. Human Health Risk Assessment for Coarse Particulate Matter. Health Canada pub. 150213.

Heavy Engineering Research Association (HERA), 2011. New Zealand steelwork corrosion and coatings guide. HERA R4-133. February 2011.

Jerrett M, Burnett RT, Ma R, Pope CA 3rd, Krewski D, Newbold KB, Thurston G, Shi Y, Finkelstein N, Calle EE, Thun MJ. (2005). Spatial analysis of air pollution and mortality in Los Angeles. Epidemiology. 2005; 16(6): 727-36.

Künzli, N, Kaiser, R et al., 2000. Public-health impact of outdoor and traffic-related air pollution: a European assessment. Lancet 356, 795-801.

Kuschel, G, Metcalfe, J, et al., 2012. Updated health and air pollution in New Zealand study; Volume 2: Technical Reports. Prepared for the Health Research Council of New Zealand, Ministry of Transport, Ministry for the Environment and New Zealand Transport Agency. March 2012.

Ministry for the Environment and Statistics New Zealand (2014). New Zealand’s Environmental Reporting Series: 2014 Air domain report. Available from www.mfe.govt.nz and www.stats.govt.nz.

Ministry for the Environment and Statistics New Zealand (2018). New Zealand’s Environmental Reporting Series: 2018. Our air 2018. October 2018. Publication reference number:  ME 1384. Available from www.mfe.govt.nz and www.stats.govt.nz.

WHO (2013). Health risks of air pollution in Europe – HRAPIE project. Recommendations for concentration–response functions for cost–benefit analysis of particulate matter, ozone and nitrogen dioxide

Source Apportionment data used

Tables A-1 to A-3 show the estimated annual mean contributions of various source profiles at various locations. This is based on monthly time series provided to NIWA by GNS Science. It should be noted that the source profile definitions (e.g. ‘soil’, ‘biomass’, etc.) are strictly defined for each dataset alone and may be inconsistent between locations and should therefore be treated as indicative rather than definitive.

The original data, methods and analyses can be found in the following reports:

Ancelet, T.; Davy, P. K.; Trompetter, W. J. 2013. Source apportionment of PM10 and PM2.5 in Nelson Airshed A, GNS Science Consultancy Report 2013/146. 95 p.

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2008. Source apportionment of airborne particles at Seaview, Lower Hutt, GNS Science Consultancy Report 2008/160

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2009. Source apportionment of airborne particles at Wainuiomata, Lower Hutt, GNS Science Consultancy Report 2009/188

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2010. Source apportionment of airborne particles at Tahunanui, Nelson, GNS Science Consultancy Report 2010/198

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2011. Source apportionment of airborne particles at Raumati, Kapiti Coast, GNS Science Consultancy Report 2011/83

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2011. Source apportionment of airborne particles in the Auckland region: 2010 analysis, GNS Science Consultancy Report 2010/262

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2011. Source apportionment of airborne particles at Dunedin, GNS Science Consultancy Report 2011/131

Davy, P. K.; Trompetter, W. J.; Markwitz, A. 2012. Source apportionment of airborne particles at Patumahoe, South Auckland, GNS Science Consultancy Report 2011/258

Wilton, E.; Trompetter, W. J. 2007. Source apportionment of PM10 in Blenheim, Prepared for Marlborough District Council

Table A-1:	Estimates of main source contributions to annual mean PM10 concentrations (g m-3) derived from data provided by GNS.

Highlighted data are winter only.

|  | Total PM10 | Biomass burning | Motor vehicles | Marine aerosol | Soil | Sulphate | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alexandra CODC | 20.0 | 20.3 | 0.7 | 0.4 | 0.5 |  | 0.0 |
| Alexandra GG | 34.5 | 26.2 | 2.3 | 0.2 | 0.2 |  | 0.0 |
| Blenheim | 10.7 | 4.4 | 1.0 | 2.3 | 1.4 | 1.0 | 0.0 |
| Dunedin | 27.2 | 3.9 | 2.8 | 5.1 | 7.4 | 1.7 | 5.1 |
| Hastings |  |  | PM2.5 only |  |  |  |  |
| Henderson | 13.7 | 2.3 | 2.0 |  | 1.2 | 1.4 | 0.3 |
| Kingsland | 15.8 | 2.9 | 2.8 | 6.8 | 1.0 | 1.4 | 0.0 |
| KPR | 18.2 | 1.5 | 5.0 | 7.6 | 1.5 | 1.7 | 0.0 |
| Lower Hutt | 16.3 | 1.1 | 2.2 | 6.3 |  | 2.4 | 0.5 |
| Masterton | 16.6 | 7.1 | 1.0 | 4.3 | 3.4 | 0.8 | 0.0 |
| Nelson A | 19.1 | 8.4 | 1.8 | 3.9 | 2.5 | 2.3 | 0.0 |
| Nelson B | 20.6 | 7.2 | 2.2 | 3.8 | 3.1 | 1.3 | 2.6 |
| Patumahoe | 10.8 | 0.9 |  | 5.3 | 2.4 | 1.1 | 0.4 |
| Penrose | 16.9 | 2.0 | 3.9 | 6.8 | 1.7 | 1.4 | 0.1 |
| Queen Str | 17.8 | 1.2 | 5.8 | 6.8 | 1.1 | 1.0 | 1.0 |
| Raumati | 22.1 | 8.9 | 1.5 | 8.5 | 1.2 | 1.6 | 0.0 |
| Takapuna | 16.2 | 2.2 | 3.4 | 6.2 | 1.0 | 2.1 | 0.5 |
| Timaru |  |  | PM2.5 only |  |  |  |  |
| Tokoroa |  | 14.7 |  | 2.5 |  | 1.8 | 1.9 |
| Upper Hutt | 11.0 | 2.3 | 1.4 | 4.0 | 2.2 | 1.3 | 0.0 |
| Wainuiomata | 13.6 | 2.9 | 1.7 | 5.6 | 1.3 | 1.2 | 0.0 |
| Whangarei | 12.5 | 4.7 | 4.6 | 1.8 |  | 0.8 | 0.0 |
| Woolston | 23.3 | 5.4 | 4.6 | 3.9 | 4.6 | 0.6 | 0.0 |

Table A-2:	Estimates of main source contributions to annual mean PM2.5 concentrations (g m-3) derived from data provided by GNS. .

Highlighted data are winter only.

|  | Total PM2.5 | Biomass burning | Motor vehicles | Marine aerosol | Soil | Sulphate | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alexandra CODC |  |  |  |  |  |  |  |
| Alexandra GG |  |  |  |  |  |  |  |
| Blenheim |  |  |  |  |  |  |  |
| Dunedin | 10.4 | 4.0 | 2.9 | 1.2 |  | 1.7 | 0.0 |
| Hastings | 11.9 | 6.9 | 1.0 | 1.2 |  | 1.1 | 0.0 |
| Henderson |  |  |  |  |  |  |  |
| Kingsland | 6.9 | 2.1 | 1.9 | 1.3 |  | 0.8 | 0.0 |
| KPR | 8.3 | 1.3 | 4.1 | 0.9 | 0.0 | 1.3 | 0.0 |
| Lower Hutt | 4.9 | 1.1 | 0.6 | 1.1 | 0.4 | 1.1 | 0.5 |
| Masterton | 7.8 | 4.7 | 0.4 | 0.7 | 0.7 | 0.6 | 0.0 |
| Nelson A | 15.1 | 10.7 | 0.7 | 1.1 | 0.9 | 1.0 | 0.0 |
| Nelson B |  |  |  |  |  |  |  |
| Patumahoe | 3.4 | 0.9 | 0.0 | 0.9 | 0.2 | 1.1 | 0.0 |
| Penrose | 7.0 | 1.5 | 2.5 | 0.5 | 0.3 | 1.0 | 0.5 |
| Queen Str | 9.3 | 0.8 | 3.6 | 2.1 | 0.1 | 1.0 | 1.0 |
| Raumati | 12.8 | 8.2 | 1.5 | 1.6 |  | 1.6 | 0.0 |
| Takapuna | 8.0 | 1.5 | 1.7 | 2.8 | 0.2 | 1.0 | 0.0 |
| Timaru | 11.8 | 5.0 | 0.9 | 2.8 |  | 0.7 | 0.8 |
| Tokoroa |  |  |  |  |  |  |  |
| Upper Hutt | 4.6 | 1.8 | 0.9 | 0.4 |  | 0.8 | 0.0 |
| Wainuiomata | 6.2 | 2.5 | 0.7 | 1.6 |  | 1.4 | 0.0 |
| Whangarei |  |  |  |  |  |  |  |
| Woolston | 9.6 | 3.2 | 2.5 | 0.5 | 0.3 | 1.2 | 0.0 |

Table A-3:	Estimates of main source contributions to annual mean PM2.5-10 concentrations (g m-3) derived from data provided by GNS

Black text is measured data. Red text is calculated from PM10 - PM2.5. Highlighted data are winter.

|  | PM2.5-10 | Wood Burning | Road Dust | Seasalt | Soil | Sulphate | Fertiliser soil |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alexandra CODC |  |  |  |  |  |  |  |
| Alexandra GG |  |  |  |  |  |  |  |
| Blenheim |  |  |  |  |  |  |  |
| Dunedin | 16.9 | 0.0 |  | 3.9 | 7.4 | 0.0 | 5.1 |
| Hastings |  |  |  |  |  |  |  |
| Henderson |  |  |  |  |  |  |  |
| Kingsland | 8.9 | 0.8 |  | 5.5 |  | 0.6 |  |
| KPR | 9.9 | 0.2 |  | 6.7 | 1.4 | 0.5 |  |
| Lower Hutt | 11.2 | -0.1 | 1.6 | 5.2 | 2.6 | 1.2 |  |
| Masterton | 7.7 | 1.5 | 0.5 | 3.3 | 1.9 | 0.1 |  |
| Nelson A | 4.0 | -2.4 |  | 2.8 | 1.7 | 1.3 |  |
| Nelson B |  |  |  |  |  |  |  |
| Patumahoe | 7.3 | 0.0 |  | 4.3 | 2.2 | 0.0 | 0.4 |
| Penrose | 9.9 | 0.6 |  | 6.3 | 1.4 | 0.4 |  |
| Queen Str | 8.5 | 0.4 |  | 4.7 | 0.9 | 0.0 |  |
| Raumati | 9.3 | 0.7 |  | 6.9 | 1.2 | 0.0 |  |
| Takapuna | 8.1 | 0.6 |  | 3.5 | 0.9 | 1.1 |  |
| Timaru |  |  |  |  |  |  |  |
| Tokoroa |  |  |  |  |  |  |  |
| Upper Hutt | 6.1 | 0.5 | 0.7 | 3.6 | 1.5 | 0.4 |  |
| Wainuiomata | 7.4 | 0.4 | 0.7 | 4.4 | 1.7 | -0.2 |  |
| Whangarei |  |  |  |  |  |  |  |
| Woolston | 13.7 | 2.2 |  | 3.4 | 4.3 | -0.6 |  |

PM10 exceedances 2015 - 2018

Table B-1 shows the annual mean PM10 concentration and number of exceedances of the National Environmental Standards for Air Quality 24-hour limit value (50 µg m-3). The data are taken from Land Air Water Aotearoa (LAWA) (https://www.lawa.org.nz/). Note: empty cells in the table result from there being no corresponding data recorded in LAWA.

Table B-1:	PM10 exceedances of NESAQ in New Zealand 2015 - 2018.

|  | Annual Mean PM10 (µg m-3) |  |  |  | Number of NES exceedances |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2018 | 2017 | 2016 | 2015 | 2018 | 2017 | 2016 | 2015 |
| Arrowtown | 18.5 | 22.2 | 19.8 | 21.4 | 30 | 45 | 32 | 29 |
| Invercargill | 21 | 20.3 | 19.8 | 20.7 | 13 | 14 | 12 | 11 |
| Richmond | 18.8 | 16.7 | 18.7 | 18 | 12 | 2 | 5 | 3 |
| Timaru | 19.5 | 21 | 23.1 | 26.2 | 8 | 17 | 27 | 26 |
| Blenheim | 17.8 | 18.9 | 17.3 | 15.1 | 7 | 11 | 4 | 4 |
| Masterton East | 14.2 | 15.2 | 15 | 14.8 | 6 | 5 | 10 | 4 |
| Awatoto | 19.7 | 18 | 18.4 | 19.6 | 4 | 1 | 0 | 3 |
| Mosgiel | 19.2 | 18.6 |  | 19.5 | 4 | 8 | 9 | 7 |
| Masterton West | 14 | 13.9 | 13.8 | 11.8 | 3 | 3 | 1 | 1 |
| Rotorua | 13.7 | 16.6 |  |  | 3 | 0 |  |  |
| Hastings | 13.5 | 13.4 | 15.3 | 14.8 | 3 | 2 | 7 | 1 |
| Christchurch (St Albans) |  |  |  |  | 3 | 4 | 3 | 4 |
| Gore | 18.5 | 18.6 | 18.4 | 19.5 | 2 | 2 | 5 | 2 |
| Nelson South StVincent | 17 | 17 | 17 | 17 | 2 | 1 | 1 | 2 |
| Kaiapoi | 16.6 | 17.4 | 17.8 | 19.6 | 2 | 10 | 7 | 13 |
| Ashburton | 16 | 17.5 | 17.8 | 19.5 | 2 | 3 | 2 | 1 |
| Tokoroa | 14.7 | 15.6 | 15.1 | 16.6 | 2 | 10 | 5 | 10 |
| Alexandra | 14.2 |  |  | 20.4 | 2 | 43 | 38 | 22 |
| Geraldine | 16 | 17 | 17 | 19 | 1 | 1 | 1 | 3 |
| Dunedin | 15 | 14.1 | 16.1 | 16.9 | 1 | 0 | 0 | 0 |
| Napier | 13 | 12.8 | 13 | 13.8 | 1 | 0 | 0 | 1 |
| Tahunanui Blackwood | 18 | 20 | 18 | 18 | 0 | 3 | 0 | 0 |
| Rangiora | 15.5 | 16.9 | 17.1 | 17.9 | 0 | 6 | 7 | 3 |
| Waimate | 14 | 16.1 | 16.2 |  | 0 | 2 | 0 |  |
| Takapuna | 13.5 | 14.3 | 13.5 | 14.9 | 0 | 1 | 3 | 0 |
| Te Kuiti | 13.4 | 14.1 | 15.2 | 15.4 | 0 | 0 | 0 | 0 |
| Wellington | 12.4 | 11.3 | 11.3 |  | 0 | 0 | 0 |  |
| Hamilton - Bloodbank | 12.1 | 12.3 | 12.5 | 12.9 | 0 | 0 | 0 | 0 |
| Nelson Centre Brook | 12 | 13 | 14 | 11 | 0 | 0 | 0 | 0 |
| Hamilton - Claudelands | 11.6 | 11.1 | 11.5 | 11.3 | 0 | 0 | 0 | 0 |
| Lower Hutt | 11.3 | 11.4 | 11.2 | 11.3 | 0 | 0 | 0 | 0 |
| Putaruru | 11.2 | 11.6 | 11.7 | 14.1 | 0 | 1 | 0 | 0 |
| Taupo | 11.2 | 13.8 | 12 | 12.5 | 0 | 1 | 0 | 0 |
| Wainuiomata | 10.9 | 11.3 | 10.6 | 10.3 | 0 | 0 | 0 | 0 |
| Tauranga | 10.7 | 9.7 | 10.2 |  | 0 | 0 | 0 |  |
| Upper Hutt | 10.4 | 10.7 | 10.5 | 10.1 | 0 | 0 | 0 | 0 |
| Thames | 5.4 | 8.6 |  |  | 0 | 0 | 0 | 0 |
| Taihape | 18.6 | 16.2 | 11.4 | 14.6 |  |  |  |  |
| Reefton | 12.4 | 13 | 16.4 | 20.2 |  |  |  |  |
| Taumaranui | 10.5 | 12 | 11.4 | 13.4 |  |  |  |  |
| Stoke Green Meadows |  | 10 | 11 |  |  | 0 | 0 |  |
| Ruakaka |  |  |  | 11.5 |  |  |  | 0 |
| Whangarei |  |  |  | 13.4 |  |  |  | 0 |
| Morrinsville |  | 11.3 |  |  |  |  | 0 |  |

The exception is Kingsland in Auckland. Marine aerosol and soil are resolved in PM10 samples here, but not in PM2.5 samples, preventing a calculation of separate contribution in the coarse fraction.

The 13th location was Dunedin where contributions from fertiliser soil were found to be greater than marine aerosol.

Although population data was collected in the 2018 census, it is not available at the required level of census area unit at the time of writing.