  

Scope One

Scope 1 emissions are direct GHG emissions occurring from sources that are owned or controlled by the company and within the organisational boundary (eg, emissions from combustion of fuel in owned or controlled vehicles).
Liquid Fuels – Stationary Combustion 

	Emission source
	Unit
	Emission factor total CO2-e
(kg CO2-e/unit)

	Diesel
	Litre
	2.66

	LPG
	Kg
	3.00

	Heavy fuel oil
	Litre
	2.99

	Light fuel oil
	Litre
	2.94


Natural Gas 

	Natural Gas
	Emission factor total CO2-e
(kg CO2-e/kWh)
	Emission factor 
total CO2-e
(kg CO2-e/GJ)

	Commercial
	0.192
	53.4

	Industry
	0.190
	52.8


Wood 

	Wood
	Emission factor total CO2-e
(kg CO2-e/kg)

	Industry
	0.0142

	Fireplaces
	0.0690


Note: The total CO2-e emission factor for wood includes CH4 and N2O emissions. This is based on ISO 14064-1 and The GHG Protocol reporting requirements for combustion of biomass as Scope 1 emissions. CO2 emissions, from the combustion of biologically sequestered carbon are reported separately.
Coal 

	Emission source
	Emission factor total CO2-e
(kg CO2-e/kg)

	Residential – Bituminous
	2.79

	Residential – Sub-bituminous
	2.06

	Residential – Lignite
	1.50

	Residential – default*
	1.92

	Commercial – Bituminous
	2.62

	Commercial – Sub-bituminous
	1.93

	Commercial – Lignite
	1.41

	Commercial – default*
	1.72

	Industry – Bituminous
	2.61

	Industry – Sub-bituminous
	1.93

	Industry – Lignite
	1.41

	Industry – default*
	2.08


*The default coal emission factor should be used if it is not possible to identify the specific type of coal. Note, the default for industrial use of coal does not include coal used by the dairy industry as it is expected this sector would know the type of coal it consumes.

Transport (liquid fuels) 

	Fuel
	Emission factor total CO2-e*
(kg CO2-e/litre)

	Regular petrol
	2.34

	Premium petrol
	2.36

	Petrol – default*
	2.34

	Diesel
	2.70

	LPG
	1.52

	Heavy fuel oil
	3.02

	Light fuel oil
	2.96


*The default petrol emission factor should be used if it is not possible to distinguish between regular and premium petrol use.

Transport (distance travelled)

	Vehicle size class
	Emission factor
total CO2-e
(kg CO2-e/km)

	Car – small
(<1600 cc)
	0.177

	Car – medium
(1600–<2500 cc)
	0.230

	Car – large
(≥ 2500 cc)
	0.301

	Car – default*
	0.230


*The default emission factor should be used if vehicle size class 
cannot be determined.

Scope Two 

Scope two emissions are indirect GHG emissions occurring from the generation of purchased electricity consumed by the company.
Purchased electricity 

	Emission source
	Emission factor total CO2-e
(kg CO2-e/kWh)

	Purchased electricity
	0.165


Scope Three 

Scope 3 emissions are other indirect GHG emissions occurring as a consequence of the activities of the company, but generated from sources not owned or controlled by the company (eg, emissions from air travel).

Air Travel
	Emission Source
	Unit
	Cabin Class
	Emission factor total CO2-e

(kg CO2-e/unit)

	Domestic
	pkm
	Average
	0.160

	Short Haul International 
(<3700 km)
	pkm
	Average
	0.0942

	 
	pkm
	Economy class
	0.0898

	 
	pkm
	Business class
	0.135

	Long Haul International 
(>3700 km)
	pkm
	Average
	0.111

	 
	pkm
	Economy class
	0.0810

	 
	pkm
	Premium economy class
	0.130

	 
	pkm
	Business class
	0.235

	 
	pkm
	First class
	0.324


Transmission and distribution line losses (purchased electricity)
	Emission source
	Emission factor total CO2-e
(kg CO2-e/kWh)

	Transmission and distribution line losses for purchased electricity
	0.0153


Transmission and distribution losses (distributed natural gas)

	Emission source
	Unit
	Emission factor total CO2-e
(kg CO2-e/unit)

	Transmission and distribution losses for distributed natural gas
	kWh
	0.0188

	
	GJ
	5.23


Taxis and rental cars

	Emission source
	Unit
	Emission factor
total CO2-e
(kg CO2-e/unit)

	Rental car – small (<1600cc)
	Km
	0.177

	Rental car – medium (1600 – <2500cc)
	Km
	0.230

	Rental car – large (≥ 2500)
	Km
	0.301

	Rental car – default*
	Km
	0.230

	Taxi travel – distance travelled
	Km
	0.301

	Taxi travel – dollars spent (GST inclusive)
	$
	0.108


*The default emission factor should be used if the vehicle size class of rental cars cannot be determined.

Waste to landfill

	Emission source
	Emission factor total CO2-e

(kg CO2e/kg)

	Landfilled waste of known composition (without landfill gas recovery)

	Paper and textiles
	2.52

	Garden and food
	0.945

	Wood
	1.89

	Landfilled waste of known composition (with landfill gas recovery)

	Paper and textiles
	0.918

	Garden and food
	0.344

	Wood
	0.688

	Landfilled waste – default values (without landfill gas recovery)

	Mixed waste (national average)
	1.00

	Office waste
	1.55

	Landfilled waste – default values (with landfill gas recovery)

	Mixed waste (national average)
	0.364

	Office waste
	0.563


Published in April 2014 
by the Ministry for the Environment. INFO 708

These greenhouse gas emission factors can be used by organisations who wish to voluntarily monitor and report greenhouse gas (GHG) emissions on an organisational basis for their New Zealand operations.


The emission factors are presented in carbon dioxide equivalents (CO2-e) for the 2012 calendar


year. Further data and information on guidance for organisations on voluntary greenhouse gas reporting can be found in the Ministry for the Environment’s publication � HYPERLINK "https://www.mfe.govt.nz/publications/climate/guidance-greenhouse-gas-reporting-2012/index.html" ��Guidance for Voluntary Corporate Greenhouse Gas Reporting� for the 2012 calendar year. 





Voluntary Greenhouse Gas Reporting �Emission Factors 2012











